1. Apollo auto

1.1 Intro

1.2 Issues

1.2.1 Road-Centric

1.2.2 Sensors

1.2.3 Computational requirements

1.2.4 Size

1.3 Conclusion

2. Nav2

2.1 additional systems

2.2 ros 2

1. Apollo auto

1.1 Intro

In their own words ”Apollo is a high performance, flexible architecture which accelerates the development, testing, and deployment of Autonomous Vehicles.”

It utilises a modular approach, shipping with eg. LiDAR, radar, camera and GNSS pipelines, that can easily be swapped and combined for the desired otucome.

A hierarchical planner and a model-predictive controller are already implemented in the Apollo project. Theese can however be tuned for low-speed, off road, etc.

Apollo offers out of the box integration with LGSVL and CARLA which allows for testing of algorithms in virtual environments, such as rough environments, ideal for the usecases of aurora

Additionally apollo have extensive dopcumentation, ready to go docker images, and a fairly active mailing list

1.2 Issues

1.2.1 Road-Centric

Apollo is primarily road centric. Whilst certain things can be swapped for our advantage, such as the lane detection system being swapped with our YoloV5 HD system, the risk will still be present, that it may reject off-road nodes all together. A potential solution to this is to replace the HD-map with a grid-based cost map, however this obviously requires extra labour

1.2.2 Sensors

Additionally apollo’s reference hardware contains multiple LiDAR’s, HQ cameras, GNSS’s etc. AURORA will only contain a single 720p camera, no lidars nor GNSS and 2 sonars. The sensor drives may not compile, and the models may not have enough data to make correct estimates

1.2.3 Computational requirements

The full stack runs on NVIDIA Xavier architecture, with a modern Intel core 3 or better processor behind it. This kind of computation will not be available on the aurora, and a decision will therefore have to be made regarding ”dumbing down” the model to work on a RPI5, or offloading the processing to an external computational device

1.2.4 Size

The code base is absolutely humongus, weighing in at around 500.000 lines. Bazel is not the most userfriendly of build systems, and there’s a metric butt ton of parameters to tune.

Creating a model there’s usable for the aurora’s exact usecase will be difficult at best

1.3 Conclusion

An algorithm this extensive, whilst with enough time and computational effort could be the ideal match for a project like aurora, is simply not feasible to implement given the limitations of hardware, budget, power and time (as deadline from todays date is about 2 months in the future)

In the following section a different approach will be suggested, that has a higher likelyhood of working, whilst not compromising too much on the base functionallity of what apollo achieved

2. Nav2

Navigation 2, commonly abbreviated as Nav2, is the  de‑facto standard ROS 2‑based framework for autonomous mobile robot  navigation. It builds upon the concepts introduced in the original ROS 1  navigation stack but leverages ROS 2’s middleware  improvements—DDS‑based communication, real‑time safety, and multi‑robot  scalability—to provide a modular, extensible, and production‑ready  solution.

It provides map-based planning, obstacle avoidance and a simple waypoint system. Computational it’s very lightweight, and designed for scenarioes just like this one

Whilst SLAM could be used to build maps on the fly, it is suggested for a PoC such as aurora, that it Nav2 sticks to prebuilt HD-maps just like Apollo would

2.1 additional systems

Ultrasonic sensors could potentially be used as a cheap depth sensor, allowing for combination of YoloV5 and RealSense D415, for an estimate of distance to detected persons, as well as aid to Nav2

2.2 ros 2

Navigation, both autonomous and manual should be fed through a ROS2 interface on the rpi, before landing on the MCU

ROS 2 is a robust, DDS‑based middleware that offers a rich set of abstractions (nodes, topics, services, actions) together with lifecycle management, security, and real‑time configurability. Its modular architecture and extensive ecosystem make it an ideal foundation for both academic research and industrial robotic applications.

Whilst ROS2 does incorporate security such as AES256, this is not recommended for a PoC as it would add additional work strain, with little to no benefit